Acrylic Clock

Production Quantity = 50

This acrylic clock has two manufactured pieces.  The background, which has a logo and an indentation for the clock insert, and the base, which simply has a groove in the top for the background to connect to it.  The processes that are currently used to produce this clock is as follows.

### Process Summary for Base

Set Up Number

Operation Description

Machine Tool Description

10

Cut-off 0.50 in. x 2.25 in. cross-sections of cast sheet acrylic stock to a length of 5.25  .015 in.

Cut-off Saw

20

Fabricate background housing on the planar surface.

3 axis vertical machining center

30

Polish all surfaces of the base

Sanding and Buffing

## Tools Currently Used for Setup 20 of Base

(Making the slot in the top of the base)

•MACHINE TOOLS: Vertical Machining Center, VF-OE

•FIXTURE: Kurt Vise with 1 ¼” parallels

•CUTTING TOOLS: 1/2 inch Double Flute End Mill

•MISCELLANEOUS TOOLS: Edge Finder, Dial Indicator

Operation Number

Operation Description

Tool Number

Tool Description

Important Process Parameters

1

Slot mill in one pass

1

1/2-inch end mill

d = .125 in.

w = 5.25 in.

V = 3000 rpm

F = 15 ipm

### CAD Drawing of Background

#### Process Summary for Background

Set Up Number

Operation Description

Machine Tool Description

10

Cut-off a 0.50 in. x 5.25 in. cross-section of cast sheet acrylic stock to a length of 8  .015 in.

Cut-off Saw

20

Machine PSU logo, fabricate clock housing, and circular contours on the planar surface.

3 axis vertical machining center

30

Polish all surfaces of the background

Sanding and Buffing

## Tools Currently Used for Setup 20 of Background

(Making the PSU logo, clock housing, and circular contours)

•MACHINE TOOLS: Vertical Machining Center, VF-OE

•FIXTURE: Kurt Vise with 1 ¼” parallels with stop

•CUTTING TOOLS: 1/32” Double Flute End Mills

•MISCELLANEOUS TOOLS: Edge Finder, Dial Indicator

d = 0.60in.

V = 2400 rpm

F = 10 ipm

1/2-inch end mill

2

End mill clock housing on the planar surface

3

d = 0.600 in.

V = 2400 rpm

F = 10 ipm

1/2-inch end mill

2

End mill circular contours on the planar surface

2

d = 0.02 in.

V = 15000 rpm

F = 17.0 ipm

1/32-inch end mill

1

End Mill PSU logo on the planar surface

1

Important Process Parameters

Tool Description

Tool Number

Operation Description

Operation Number

#### Final Assembly of the Clock

•Join the base and the background by press fitting and acrylic glue

•Insert battery into clock insert

•Place clock insert into clock housing